# 6. EPFD Calculation Input Parameters Specification

## 6.1	Introduction

### 6.1.1 Purpose

This document specifies the complete set of input parameters required for EPFD calculation for non-GSO (non-Geostationary Satellite Orbit) satellite systems subject to Article 22 EPFD limits (RR Nos. 22.5C, 22.5D, 22.5F, and 22.5L).

### 6.1.2. Scope

The specification covers:

- Database table structures in SNS (Space Network System) database

- XML format for mask data submission

- Scenario-based operational parameter management

- Orbital configuration management

- Mask linking mechanisms

### 6.1.3 Replaced Tables

Note: The new scenario-based structure replaces the following legacy tables:

- grp table (group parameters)

- non_geo table (non-GSO specific parameters)

These tables are not documented in this specification as they are superseded by the new structure.

More details on migration from the structure before SNS v10 can be found in Appendix 1B.

## 6.2. Database Structure Overview

### 6.2.1 Core Tables Summary

The SNS v10 structure includes the following tables for EPFD calculation:

| Table Name | Purpose |
| --- | --- |
| epfd_param | Core operating parameters for each examination scenario |
| epfd_freq | Frequency ranges associated with examination scenarios |
| sat_oper | Satellite operational parameters by latitude range |
| mask_info | PFD/EIRP mask metadata and descriptors |
| mask_lnk1 | Links PFD and EIRP SS masks to scenarios/orbital planes (satellites) |
| mask_lnk2 | Links EIRP ES masks to scenarios/earth stations/orbital planes (satellites) |
| mask_lnk3 | Links operating parameters masks to notices |
| orbit | Orbital parameters including orbital configuration identifiers |
| phase | Satellite positions within orbital plane |

## 6.3. Scenario-Based Parameter Management

### 6.3.1 Examination Scenario Concept

An examination scenario defines an individual operational scenario sharing the same set of operating parameters applicable to specific frequency ranges. Multiple scenarios can be submitted for different frequency ranges or operational modes, but only one scenario per frequency range and orbital configuration (as per current requirement of S.1503-4 it may change with future revisions).

### 6.3.2 epfd_param Table

The epfd_param table contains the core operating parameters for each examination scenario.

#### 6.3.2.1 Table Structure

| Field Name | Data Type | Units | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number(9) | - | Unique identifier of the notice | Primary Key, NOT NULL |
| scen_id | Number(9) | - | Identifier of the examination scenario | Primary Key, NOT NULL, > 0 |
| act_code | Text (1) | - | Code indicating the action to be taken on the entity (“A” – add, “M” – modify, “S”- suppress) | NULL, or for MOD “A” or ”M” or ”S” |
| scen_name | Text(50) | - | Descriptive name of the scenario | NOT NULL |
| orbit_set_id | Number(4) | - | Unique orbital configuration number (see §3.2.2) | Optional, can be NULL |
| nbr_sat_td | Number(9) | - | Maximum number of non-GSO satellites receiving simultaneously (Nco) with overlapping frequencies from associated earth stations within a given cell | ≥ 0 or NULL if operating parameter mask is used or for all associated frequency bands in epfd_freq, emi_rcp <> “R” |
| density | Number(9) | 1/km² | Average number of associated earth stations transmitting with overlapping frequencies per km² in a cell | > 0 or NULL if operating parameter mask is used or if for all associated frequency bands in epfd_freq, emi_rcp <> “R” |
| avg_dist | Number(9) | km | Average distance between co-frequency cells | ≥ 0 (0 means only one ES in GSO footprint) or NULL if operating parameter mask is used or if for all associated frequency bands in epfd_freq, emi_rcp <> “R” |
| f_x_zone | Text(1) | - | Type of exclusion zone (“Y” = based on topocentric angle α only) | “Y” or NULL if operating parameter mask is used |
| x_zone | Number(9) | degrees | Width of the exclusion zone (α₀) | ≥ 0 or NULL if operating parameter mask is used |
| elev_min | Number(9) | degrees | Minimum elevation angle (ε₀) at which any associated earth station can transmit to or receive from a satellite | ≥ 0, ≤ 90 or NULL if operating parameter mask is used |

#### 6.3.2.2 Orbital Configuration Identifier (orbit_set_id)

The orbit_set_id parameter indicates a unique orbital configuration number:

- Purpose: Allows scenarios to be associated with specific orbital configurations when multiple configurations exist

- Usage: Referenced in the orbit table to identify which satellites belong to which configuration

- NULL Value Behavior: When orbit_set_id is NULL or empty in epfd_param, the scenario applies to all orbital configurations in the notice

- Non-NULL Behavior: When specified, the scenario applies only to satellites in the orbital plane(s) with the matching orbit_set_id

Example Use Cases:

1. Different operational parameters for different orbital shells

2. Multiple constellation types within a single filing

#### 6.3.2.3 Action code (act_code)

The act_code parameter indicates a code (A, M, S) indicating the action to be taken on the scenario:

- Purpose:

- Identify added, modified or suppressed scenarios

- Identify scenarios to be considered in examination of MOD

- Usage: To be provided only for MOD for the non-GSO satellite network subject to Article 22 EPFD limits

- Action codes “A”, “M” Behavior: When specified, the scenarios were added or modified and should be considered in examination

- Action code “S” Behavior: When act_code = “S”, the scenario will be suppressed and should not be considered in examination

#### 6.3.2.4 Scenario Types and Examples

Common scenario types include:

- User Terminal Uplink Operations

  - Different earth station distributions (density, average distance)

  - Specific exclusion zone angles for user terminals

- Gateway Station Uplink Operations

  - Different density and distribution patterns

  - May have different exclusion zones

- User Terminal Downlink Operations

  - Specific Nco values for user link

  - Different latitude-dependent operational parameters

- Gateway Station Downlink Operations

  - Different operational characteristics

  - May operate in different frequency ranges

- Multiple Orbital Configurations

  - Different scenarios for different orbital sets

  - Each linked via orbit_set_id

### 6.3.3 epfd_freq Table

The epfd_freq table contains frequency ranges associated with examination scenarios.

#### 6.3.3.1 Table Structure

| Field Name | Data Type | Units | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number(9) | - | Unique identifier of the notice | Foreign Key, NOT NULL |
| scen_id | Number(9) | - | Identifier of the examination scenario | Foreign Key, NOT NULL |
| seq_no | Number(4) | - | Sequence number for multiple frequency ranges | Primary Key, > 0 |
| emi_rcp | Text(1) | - | Direction of emission (“E” = Earth-to-space, “R” = space-to-Earth) | “E” or “R”, NOT NULL |
| freq_min | Number(12) | MHz | Minimum frequency of the range | > 0, NOT NULL, only within bands subject to Article 22 EPFD limits |
| freq_max | Number(12) | MHz | Maximum frequency of the range | > freq_min, NOT NULL, only within bands subject to Article 22 EPFD limits |

#### 6.3.3.2 Frequency Range Association

Each scenario must have at least one associated frequency range. Multiple frequency ranges can be associated with a single scenario if the operational parameters are identical across those ranges.

## 6.4. Orbital Configuration Parameters

### 6.4.1 orbit Table

The orbit table defines the orbital parameters for each satellite or orbital plane.

#### 6.4.1.1 Enhanced Table Structure

| Field Name | Data Type | Units | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number(9) | - | Unique identifier of the notice | Foreign Key, NOT NULL |
| orb_id | Number(2) | - | Sequence number of the orbital plane | Primary Key, NOT NULL, > 0 |
| act_code | Text (1) | - | Code indicating the action to be taken on the entity (“A” – add, “M” – modify, “S”- suppress) | NULL, or for MOD NULL or “A” or ”M” or ”S” |
| orbit_set_id | Number(4) | - | Unique orbital configuration identifier | Optional |
| nbr_sat_pl | Number(2) | - | Number of satellites per orbital plane | NOT NULL, > 0 Not actually used and can be ignored |
| inclin_ang | Number(5) | degrees | Inclination angle of satellite orbit | NOT NULL, 0-180 |
| apog_km | Number(10) | km | Apogee altitude above Earth’s surface | NOT NULL, > 0 |
| perig_km | Number(10) | km | Perigee altitude above Earth’s surface | NOT NULL, > 0, ≤ apog_km |
| perig_arg | Number(5) | degrees | Angular separation between ascending node and perigee | If apog_km <> perig_km, 0-360 |
| op_ht_km | Number(10) | km | Minimum operating height above Earth’s surface | NOT NULL, > 0 |
| f_stn_keep | Text(1) | - | Flag indicating station-keeping usage (“Y” = Yes, “N” = No) | “Y” or “N”, NOT NULL |
| rpt_prd_dd | Number(3) | days | Day component of constellation repeat period | If f_stn_keep = “Y” |
| rpt_prd_hh | Number(2) | hours | Hour component of constellation repeat period | If f_stn_keep = “Y” |
| rpt_prd_mm | Number(2) | minutes | Minute component of constellation repeat period | If f_stn_keep = “Y” |
| rpt_prd_ss | Number(2) | seconds | Second component of constellation repeat period | If f_stn_keep = “Y” |
| f_precess | Text(1) | - | Flag for specific precession rate (“Y” = Yes, “N” = No) | “Y” or “N”, NOT NULL |
| precession | Number(6) | degrees/day | Precession rate of ascending node | If f_precess = “Y”, ≥ 0 |
| long_asc | Number(6) | degrees | Longitude of ascending node | NOT NULL, 0-360 |
| keep_rnge | Number(4) | degrees | Longitudinal tolerance of LAN | NOT NULL,≥ 0 |

#### 6.4.1.2 orbit_set_id Usage in orbit Table

The orbit_set_id field in the orbit table serves to group orbital planes into logical configurations:

Purpose:

- Groups multiple orbital planes that constitute a single operational configuration

- Enables scenarios to target specific orbital configurations

- Supports phased deployments and multiple constellation types

Usage Patterns:

- Single Configuration: All orbital planes have the same orbit_set_id

- Multiple Configurations: Different orbital planes have different orbit_set_id values

- NULL Values: If orbit_set_id is NULL, the orbital plane is part of a default configuration

Example:

orbit_set_id = 1: Low shell (550 km) with 10 orbital planes
orbit_set_id = 2: Medium shell (1200 km) with 5 orbital planes
orbit_set_id = NULL: All orbital planes (default)

#### 6.4.1.3 act_code Usage in orbit Table

The act_code parameter indicates a code (A, M, S) indicating the action to be taken on the orbital plane:

- Purpose:

- Identify added, modified or suppressed orbital planes

- Identify orbital planes to be considered in examination of MOD

- Usage: To be provided only for MOD

- Action codes NULL, “A”, “M” Behavior: When specified, the orbital planes were maintained, added or modified and should be considered in examination

- Action code “S” Behavior: When act_code = “S”, the orbital plane will be suppressed and should not be considered in examination

### 6.4.2 phase Table

The phase table defines the initial position of satellites within orbital planes.

#### 6.4.2.1 Table Structure

| Field Name | Data Type | Units | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number(9) | - | Unique identifier of the notice | Foreign Key, NOT NULL |
| orb_id | Number(2) | - | Sequence number of the orbital plane | Foreign Key, NOT NULL, > 0 |
| orb_sat_id | Number(2) | - | Satellite sequence number in the orbital plane | Primary Key, NOT NULL, > 0 |
| phase_ang | Number(5) | degrees | Initial phase angle of the satellite in the orbital plane | 0-360, phase angle is measured from the point of the ascending node |

## 6.5. Mask Parameters and Linking

### 6.5.1 mask_info Table

The mask_info table contains metadata for all PFD and EIRP masks.

#### 6.5.1.1 Table Structure

| Field Name | Data Type | Units | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number(9) | - | Unique identifier of the notice | Foreign Key, NOT NULL |
| mask_id | Number(4) | - | Unique identifier of the mask | Primary Key, NOT NULL |
| freq_min | Number(12) | MHz* | Lowest frequency for which the mask is valid | > 0, NOT NULL |
| freq_max | Number(12) | MHz* | Highest frequency for which the mask is valid | > freq_min, NOT NULL |
| f_mask | Text(1) | - | Type of mask | “P”, “E”, “S”, or “R”, NOT NULL |
| f_mask_type | Text(1) | - | Format type of the PFD/EIRP mask | If f_mask = “S”,“O”, if f_mask = “E”, “O” or “D”, if f_mask = “P”, “A” or “Z”, if f_mask = “R”, NULL |

#### * In SNS v10 it is currently GHz, care should be taken to introduce this change in new SNS structure.

#### 6.5.1.2 Mask Types (f_mask)

| Code | Mask Type | Description | Direction |
| --- | --- | --- | --- |
| P | PFD mask | Power Flux-Density mask for space-to-Earth | Downlink (E) |
| E | EIRP ES mask | Earth Station EIRP mask for Earth-to-space | Uplink (R) |
| S | EIRP SS mask | Space Station EIRP mask for inter-satellite | Inter-satellite (E) |
| R | Operating parameters mask | Additional operating parameters in XML format | Both |

#### 6.5.1.3 Mask Format Types (f_mask_type)

| Code | Format Type | Parameters | Description |
| --- | --- | --- | --- |
| O | Off-axis angle | (θ, latitude) | E.i.r.p. or PFD as function of off-axis angle and latitude |
| D | Azimuth/Elevation/DeltaLong | (azimuth, elevation, ΔLong, latitude) | E.i.r.p. as function of azimuth, elevation, delta longitude, and latitude |
| A | Alpha angle | (α, ΔLongitude, latitude) | PFD as function of alpha angle, longitude difference, and latitude |
| Z | Azimuth/Elevation | (azimuth, elevation, latitude) | PFD as function of azimuth, elevation, and latitude |

### 6.5.2 mask_lnk1 Table

Links PFD masks and EIRP SS masks to examination scenarios and satellites.

#### 6.5.2.1 Table Structure

| Field Name | Data Type | Description | Validation |
| --- | --- | --- | --- |
| ntc_id | Number(9) | Unique identifier of the notice | Foreign Key, NOT NULL |
| scen_id | Number(9) | Identifier of the examination scenario | Foreign Key, NOT NULL |
| seq_no | Number(4) | Sequence number | Primary Key, > 0 |
| orb_id | Number(2) | Identifier of the orbital plane | NOT NULL and > 0 or -1 if applies to all orbital planes, |
| sat_orb_id | Number(2) | Identifier of the satellite in the orbital plane | NULL (for orb_id = -1) or > 0 if applies to specific satellites or -1 (for orb_id > 0) if applies to all satellites in orbital plane |
| mask_id | Number(4) | Identifier of the mask | Foreign Key to mask_info, NOT NULL |

#### 6.5.2.2 Linking Logic

Satellite Selection:

- orb_id = -1 AND sat_orb_id = NULL: Mask applies to all satellites in all orbital planes

- orb_id = N AND sat_orb_id = -1: Mask applies to all satellites in orbital plane N

- orb_id = N AND sat_orb_id = M: Mask applies to satellite M in orbital plane N

Scenario Association:

- Each mask link is associated with a specific scen_id

- Masks can be reused across multiple scenarios by creating multiple link entries

### 6.5.3 mask_lnk2 Table

Links EIRP ES masks to examination scenarios, earth stations, and satellites.

#### 6.5.3.1 Table Structure

| Field Name | Data Type | Description | Validation |
| --- | --- | --- | --- |
| ntc_id | Number(9) | Unique identifier of the notice | Foreign Key, NOT NULL |
| scen_id | Number(9) | Identifier of the examination scenario | Foreign Key, NOT NULL |
| seq_no | Number(4) | Sequence number | Primary Key, ≥ 0 |
| e_as_id | Number(4) | Identifier of the associated earth station | > 0 or -1 if applies to all ES |
| orb_id | Number(2) | Identifier of the orbital plane | NOT NULL and > 0 or -1 if applies to all orbital planes, |
| sat_orb_id | Number(2) | Identifier of the satellite in the orbital plane | NULL (for orb_id = -1) or > 0 if applies to specific satellites or -1 (for orb_id > 0) if applies to all satellites in orbital plane |
| mask_id | Number(4) | Identifier of the mask | Foreign Key to mask_info, NOT NULL |

#### 6.5.3.2 Linking Logic

Earth Station Selection:

- e_as_id = -1: Mask applies to all earth stations (typical ES)

- e_as_id = N: Mask applies to specific earth station N

Satellite Selection:

- Same logic as mask_lnk1 (see §5.2.2)

Usage:

- For typical earth stations using density-based distribution, set e_as_id = -1

- For specific earth stations (e.g., gateways), reference the specific e_as_id from e_as_stn table

### 6.5.4 mask_lnk3 Table

Links operating parameters masks (XML format) to notices.

#### 6.5.4.1 Table Structure

| Field Name | Data Type | Description | Validation |
| --- | --- | --- | --- |
| ntc_id | Number(9) | Unique identifier of the notice | Foreign Key, NOT NULL |
| param_id | Number(4) | Identifier of the operating parameters mask | Foreign Key to mask_info, NOT NULL |

#### 6.5.4.2 Purpose

Operating parameters masks provide extended parameters in XML format including:

- Minimum elevation angles by latitude and azimuth

- Exclusion zone angles by latitude and orbital plane

- Maximum co-frequency satellite tracking parameters

- Minimum tracking duration by latitude

- Additional operational constraints”

When operational parameters mask is used certain parameters can be provided as empty in epfd_param table.

## 6.6. Satellite transmission parameters table (Nco)

### 6.6.1 sat_oper Table

The sat_oper table contains satellite transmission parameters that vary by latitude range, associated with specific examination scenarios. Its purpose is to provide maximum number of non-GSO satellites transmitting with overlapping frequencies (Nco) to a specific location within the latitude range.

#### 6.6.1.1 Table Structure

| Field Name | Data Type | Units | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number(9) | - | Unique identifier of the notice | Foreign Key, NOT NULL |
| scen_id | Number(9) | - | Identifier of the examination scenario | Foreign Key, NOT NULL, except scen_id for which all associated frequency bands in epfd_freq table with emi_rcp <> “E” |
| lat_fr | Number(9) | degrees | Start of the latitude range | -90 ≤ lat_fr < lat_to |
| lat_to | Number(9) | degrees | End of the latitude range | lat_fr < lat_to ≤ 90 |
| nbr_op_sat | Number(9) | - | Maximum number of non-GSO satellites transmitting with overlapping frequencies (Nco) to a given location within the latitude range | ≥ 0 |

#### 6.6.1.2 Purpose

This table enables latitude-dependent transmission parameters, recognizing that satellite visibility and operational characteristics vary with latitude:

- Polar regions may have different satellite visibility than equatorial regions

- Coverage patterns differ based on orbital inclination

- Operational modes may vary by service area latitude

#### 6.6.1.3 Usage Guidelines

Latitude Range Definition:

- Ranges should be contiguous to cover the entire operational latitude range (-90° to 90°)

- Ranges must not overlap

- Typical ranges: polar (60° to 90°), mid-latitude (30° to 60°), equatorial (-30° to 30°)

nbr_op_sat Values:

- Must reflect the maximum number of satellites that can serve a location at that latitude

- For latitude ranges where no service is provided nbr_op_sat=0

- Consider orbital geometry and coverage patterns

- Account for frequency reuse and beam steering capabilities

#### 6.6.1.4 Relationship with scen_id

The scen_id field in sat_oper references the scen_id in the epfd_param table, creating a direct association:

epfd_param (scen_id=1, scenario="GAteway Terminal Downlink")
    ↓
sat_oper (scen_id=1, lat_fr=-90, lat_to=-60, nbr_op_sat=3)
sat_oper (scen_id=1, lat_fr=-60, lat_to=60, nbr_op_sat=5)
sat_oper (scen_id=1, lat_fr=60, lat_to=90, nbr_op_sat=3)

This allows:

- Different Nco values for different latitudes within a single scenario

- Accurate modeling of constellation behavior across the globe

## 6.7. XML Mask Format Specification

### 6.7.1 Overview

Masks and extended operating parameters are submitted in XML format, allowing for complex multi-dimensional data structures.

### 6.7.2 XML Structure for Operating Parameters

#### 6.7.2.1 Header Format

<?xml version="1.0"?>
<satellite_system sat_name="MySatName" ntc_id="123456789">
  <non_gso_operating_parameters 
      es_lat_max="+90" 
      es_lat_min="-90" 
      es_distance="200" 
      es_density="0.00001" 
      c_name="orb_id" 
      b_name="azimuth" 
      a_name="latitude" 
      high_freq_mhz="12000" 
      low_freq_mhz="10000" 
      param_id="1" 
      min_angle_at_es="5" 
      min_angle_at_sat="5"
      max_co_freq="2"
      max_co_freq_sat="2"
      min_duration="400">

#### 6.7.2.2 Parameter Definitions

| Parameter | Type | Units | Description |
| --- | --- | --- | --- |
| ntc_id | Integer | - | Notice identifier |
| sat_name | String | - | Satellite network name |
| param_id | Integer | - | Operating parameters identifier |
| low_freq_mhz | Double | MHz | Minimum frequency |
| high_freq_mhz | Double | MHz | Maximum frequency |
| a_name | Enum | - | “latitude” (range: -90 to +90) |
| b_name | Enum | - | “azimuth” (range: 0 to 360) |
| c_name | Enum | - | “orb_id” (range: 1 to 9999, 0 = all orbits) |
| es_density | Double | 1/km² | Earth station density (>0 and <=1) (see section D5.2.5) |
| es_distance | Double | km | Distance between ES/beam centers (>=0 and <9999) 0 assumes special case when only ES is populated within GSO boresight (see. D5.2.5). |
| es_lat_min | Double | degrees | Minimum ES latitude (-90 to 90). Should be less than[or equal] es_lat_max. |
| es_lat_max | Double | degrees | Maximum ES latitude (-90 to 90). Should be greater[or equal] than es_lat_min. |
| min_angle_at_sat | Double | degrees | Minimum angle at satellite between ESs (0-90) |
| min_angle_at_es | Double | degrees | Minimum angle at ES between satellites (0-90) |
| max_co_freq | Integer | - | Maximum co-frequency satellites at ES (0-9999) |
| max_co_freq_sat | Integer | - | Maximum co-frequency ESs at satellite (0-9999) |
| min_duration | Integer | seconds | Minimum satellite tracking duration (1-99999) |
| elev_angle | Double | degrees | Minimum elevation angle (0-90) |

#### 6.7.2.3 Exclusion Zone Array

<min_exclude c="1">
      <exclusion_zone_angle a="-75">0</exclusion_zone_angle>
      <exclusion_zone_angle a="-45">3</exclusion_zone_angle>
      <exclusion_zone_angle a="-15">5</exclusion_zone_angle>
      <exclusion_zone_angle a="15">5</exclusion_zone_angle>
      <exclusion_zone_angle a="45">3</exclusion_zone_angle>
      <exclusion_zone_angle a="75">0</exclusion_zone_angle>
    </min_exclude>

Parameters:

- c attribute: Orbital plane identifier (orb_id)

- a attribute: Latitude in degrees

- Element value: Exclusion zone angle in degrees (double, >0)

#### 6.7.2.4 Maximum Co-Frequency Array

<max_co_freq a="0">2</max_co_freq>

Parameters:

- a attribute: Latitude in degrees

- Element value: Maximum number of co-frequency satellites (

#### 6.7.2.5 Minimum Duration Array

<min_duration a="-50">400</min_duration>
    <min_duration a="0">1000</min_duration>
    <min_duration a="50">400</min_duration>

Parameters:

- a attribute: Latitude in degrees

- Element value: Minimum tracking duration in seconds

#### 6.7.2.6 Minimum Elevation Angle Array

<min_elev a="-30">
      <elev_angle b="0">30</elev_angle>
      <elev_angle b="90">40</elev_angle>
      <elev_angle b="180">30</elev_angle>
      <elev_angle b="270">40</elev_angle>
    </min_elev>

Parameters:

- a attribute: Latitude in degrees

- b attribute: Azimuth in degrees (0-360)

- Element value: Minimum elevation angle in degrees

### 6.7.3 XML Structure for PFD Masks

#### 6.7.3.1 Alpha/DeltaLongitude Format

<satellite_system ntc_id="123456789" sat_name="MySatName">
  <pfd_mask 
      mask_id="3" 
      low_freq_mhz="10000" 
      high_freq_mhz="40000" 
      refbw_khz="40" 
      type="alpha_deltaLongitude" 
      a_name="latitude" 
      b_name="alpha" 
      c_name="deltaLongitude">
    <by_a a="0">
      <by_b b="-180">
        <pfd c="-20">-150</pfd>
        <pfd c="0">-140</pfd>
        <pfd c="20">-150</pfd>
      </by_b>
      <by_b b="-8">
        <pfd c="-20">-165</pfd>
        <pfd c="0">-155</pfd>
        <pfd c="20">-165</pfd>
      </by_b>
    </by_a>
  </pfd_mask>
</satellite_system>

#### 6.7.3.2 Azimuth/Elevation Format

<pfd_mask 
    mask_id="4" 
    low_freq_mhz="17300" 
    high_freq_mhz="17700" 
    refbw_khz="40" 
    type="azimuth_elevation" 
    a_name="latitude" 
    b_name="azimuth" 
    c_name="elevation">
  <by_a a="45">
    <by_b b="0">
      <pfd c="10">-145</pfd>
      <pfd c="30">-155</pfd>
      <pfd c="60">-170</pfd>
    </by_b>
  </by_a>
</pfd_mask>

### 6.7.4 XML Structure for EIRP ES Masks

#### 6.7.4.1 Off-Axis Angle Format (format=“T”)

<satellite_system ntc_id="123456789" sat_name="MySatName">
  <eirp_mask_es 
      mask_id="1" 
      low_freq_mhz="10000" 
      high_freq_mhz="12000" 
      refbw_khz="40" 
      format="T" 
      a_name="latitude" 
      b_name="offaxis angle" 
      ES_ID="-1">
    <by_a a="0">
      <eirp b="0">30.0206</eirp>
      <eirp b="1">20.0206</eirp>
      <eirp b="2">12.49485</eirp>
      <eirp b="5">2.54634976</eirp>
      <eirp b="10">-4.9794</eirp>
      <eirp b="180">-18.9471149</eirp>
    </by_a>
  </eirp_mask_es>
</satellite_system>

Note: ES_ID="-1" indicates typical ES (density-based). For specific ES, use the actual ES identifier.

#### 6.7.4.2 Azimuth/Elevation/DeltaLong Format (format=“A”)

<eirp_mask_es 
    mask_id="2" 
    low_freq_mhz="14000" 
    high_freq_mhz="14500" 
    refbw_khz="40" 
    format="A" 
    a_name="latitude" 
    c_name="azimuth angle" 
    d_name="elevation angle" 
    e_name="DeltaLongES" 
    ES_ID="-1">
  <by_a a="0">
    <by_c c="0">
      <by_d d="10">
        <eirp e="0">30.0206</eirp>
        <eirp e="5">28.0206</eirp>
        <eirp e="10">25.0206</eirp>
      </by_d>
    </by_c>
  </by_a>
</eirp_mask_es>

### 6.7.5 XML Structure for EIRP SS Masks

<satellite_system ntc_id="123456789" sat_name="MySatName">
  <eirp_mask_ss 
      mask_id="5" 
      low_freq_mhz="17300" 
      high_freq_mhz="17700" 
      refbw_khz="40" 
      a_name="latitude" 
      b_name="offaxis angle">
    <by_a a="0">
      <eirp b="0">30.0206</eirp>
      <eirp b="1">20.0206</eirp>
      <eirp b="5">2.54634976</eirp>
      <eirp b="180">-18.9471149</eirp>
    </by_a>
  </eirp_mask_ss>
</satellite_system>

## 6.8. Parameter Validation Rules

### 6.8.1 Database Integrity Rules

#### 6.8.1.1 Scenario Validation

- Each scenario must have at least one frequency range in epfd_freq

- Each scenario associated with downlink transmission must have associated Nco parameters in sat_oper

- Scenarios must have appropriate masks linked via mask_lnk1, mask_lnk2, or both

- scen_id must be unique within a notice

- If orbit_set_id is specified in epfd_param, corresponding orbital planes must exist in orbit table with matching orbit_set_id

#### 6.8.1.2 Orbital Configuration Validation

- orbit_set_id in orbit table must be consistent across related orbital planes

- At least one orbital plane must exist for each orbit_set_id referenced in epfd_param

- If orbit_set_id is NULL in epfd_param, at least one orbital plane must exist

- Altitude parameters: perig_km ≤ apog_km

- Operating height: perig_km ≤ op_ht_km ≤ apog_km

#### 6.8.1.3 Mask Linking Validation

- Each mask referenced in mask_lnk1, mask_lnk2, or mask_lnk3 must exist in mask_info

- Mask frequency ranges must overlap with scenario frequency ranges

- PFD masks (f_mask=‘P’) can only be linked via mask_lnk1

- EIRP ES masks (f_mask=‘E’) can only be linked via mask_lnk2

- EIRP SS masks (f_mask=‘S’) can only be linked via mask_lnk1

- Operating parameter masks (f_mask=‘R’) can only be linked via mask_lnk3

### 6.8.2 Operational Parameter Ranges

#### 6.8.2.1 epfd_param Table

| Parameter | Minimum | Maximum | Special Values |
| --- | --- | --- | --- |
| nbr_sat_td | 0 | 9999 | 0 = no uplink tracking |
| density | >0 | 1.0 | Typical: 0.00001 - 0.001 |
| avg_dist | 0 | 99999 | 0 = single ES in footprint |
| x_zone | 0 | 90.0 | Typical: 3-15 degrees |
| elev_min | 0 | 90 | Typical: 10-40 degrees |

#### 6.8.2.2 sat_oper Table

| Parameter | Minimum | Maximum | Notes |
| --- | --- | --- | --- |
| lat_fr | -90 | <lat_to | Must be less than lat_to |
| lat_to | >lat_fr | 90 | Must be greater than lat_fr |
| nbr_op_sat | 0 | 9999 | At least 1 satellite required for at least 1 latitude range |

#### 6.8.2.3 orbit Table

| Parameter | Minimum | Maximum | Notes |
| --- | --- | --- | --- |
| inclin_ang | 0 | 180 | Degrees |
| apog_km | 160 | 999999 | Minimum ~LEO altitude |
| perig_km | 160 | apog_km | Must be ≤ apog_km |
| op_ht_km | 160 | perig_km | Must be ≤ perig_km |
| long_asc | 0 | <360 | Degrees |
| keep_rnge | 0 | 180 | Degrees |

### 6.8.3 XML Validation Rules

#### 6.8.3.1 General Rules

- XML must be well-formed and valid against schema

- All required attributes must be present

- Numeric values must be within specified ranges

- Array indices must be monotonically increasing (for interpolation)

#### 6.8.3.2 Mask-Specific Rules

PFD Masks:

- Reference bandwidth must match Article 22 requirements (in case several bandwidths are present for the same range/type of the limit, it should be provided in one of the ref. BW)

- All components should be in appropriate range (see above).

EIRP ES Masks:

- Must be monotonically decreasing for format=“T”

- All components should be in appropriate range (see above).

- Reference bandwidth must match Article 22 requirements (in case several bandwidths are present for the same range/type of the limit, it should be provided in one of the ref. BW)

EIRP SS Masks:

- All components should be in appropriate range (see above).

- Reference bandwidth must match Article 22 requirements (in case several bandwidths are present for the same range/type of the limit, it should be provided in one of the ref. BW)

Operating Parameters Masks:

- Latitude arrays must cover ES operational range (es_lat_min to es_lat_max)

- Azimuth values must be in range [0, 360]

- That there is only a single non-GSO system operating parameter set per frequency range.

That there is a non-GSO system operating parameters set for each frequency range that is to be examined.

That if the MIN_EXCLUDE varies by orbit plane, that a value is defined for each orbit plane

MIN_EXCLUDE[Latitude] ≥ 0

MIN_ELEV[Latitude, Azimuth] ≥ 0

MIN_DURATION[Latitude] ≥ 1 second

MAX_CO_FREQ[Latitude] ≥ 0

ES_DENSITY > 0

ES_DISTANCE => 0 (ES_DISTANCE = 0 means that only one earth station can be distributed inside the victim GSO footprint)

+90º > ES_LAT_MIN ≥ −90º

+90º ³ ES_LAT_MAX > −90º

ES_LAT_MAX > ES_LAT_MIN

MIN_ANGLE_AT_ES ≥ 0

MIN_ANGLE_AT_SAT≥ 0

MAX_CO_FREQ_SAT ≥ 0

## Appendix A1: Complete Example Scenario

### A.1 Scenario Description

System: LEO constellation with two orbital configurations

- Configuration 1 (orbit_set_id=1): Low shell at 550 km - 10 orbital planes, 60 satellites per plane - Inclination: 53 degrees

- Configuration 2 (orbit_set_id=2): High shell at 1200 km - 5 orbital planes, 40 satellites per plane - Inclination: 70 degrees

Scenarios:

  - User terminal uplink (Ka-band) Configuration 1

  - User terminal uplink (Ka-band) Configuration 2

3.	Gateway uplink (Ka-band)  Both configurations

### A.2 Database Entries

#### epfd_param Table

ntc_id=123456789, scen_id=1, scen_name="User Terminal Uplink Cfg1", orbit_set_id=1,
  nbr_sat_td=2, density=0.00001, avg_dist=200, f_x_zone='Y', 
  x_zone=10, elev_min=25

ntc_id=123456789, scen_id=2, scen_name="User Terminal Uplink Cfg2", orbit_set_id=2,
  nbr_sat_td=2, density=0.000001, avg_dist=100, f_x_zone='Y', 
  x_zone=10, elev_min=25

ntc_id=123456789, scen_id=3, scen_name="Gateway Uplink", orbit_set_id=NULL,
  nbr_sat_td=4, density=0.000001, avg_dist=1000, f_x_zone='Y', 
  x_zone=5, elev_min=10

#### epfd_freq Table

ntc_id=123456789, scen_id=1, seq_no=1, emi_rcp='R', freq_min=27500, freq_max=28600
ntc_id=123456789, scen_id=2, seq_no=1, emi_rcp='R', freq_min=27500, freq_max=28600
ntc_id=123456789, scen_id=3, seq_no=2, emi_rcp='R', freq_min=29500, freq_max=30000

orbit Table

ntc_id=123456789, orb_id=1, orbit_set_id=1, inclin_ang=53.0,
  apog_km=550, perig_km=550, op_ht_km=540, ...
  
ntc_id=123456789, orb_id=2, orbit_set_id=1, inclin_ang=53.0,
  apog_km=550, perig_km=550, op_ht_km=540, ...
  
[... orb_id=3 through 10 for Configuration 1 ...]

ntc_id=123456789, orb_id=11, orbit_set_id=2, inclin_ang=70.0,
  apog_km=1200, perig_km=1200, op_ht_km=1190, ...
  
[... orb_id=12 through 15 for Configuration 2 ...]

#### sat_oper Table

ntc_id=123456789, scen_id=1, lat_fr=-90, lat_to=-60, nbr_op_sat=2
ntc_id=123456789, scen_id=1, lat_fr=-60, lat_to=60, nbr_op_sat=4
ntc_id=123456789, scen_id=1, lat_fr=60, lat_to=90, nbr_op_sat=2

ntc_id=123456789, scen_id=2, lat_fr=-90, lat_to=90, nbr_op_sat=3

ntc_id=123456789, scen_id=3, lat_fr=-90, lat_to=-60, nbr_op_sat=2
ntc_id=123456789, scen_id=3, lat_fr=-60, lat_to=60, nbr_op_sat=5
ntc_id=123456789, scen_id=3, lat_fr=60, lat_to=90, nbr_op_sat=2

#### mask_info Table

ntc_id=123456789, mask_id=1, freq_min=27500, freq_max=28600, f_mask='E', f_mask_type='O'
ntc_id=123456789, mask_id=1, freq_min=29500, freq_max=30000, f_mask='E', f_mask_type='O'

#### mask_lnk1 Table (PFD and EIRP SS masks)

no masks (since only uplink)

#### mask_lnk2 Table (EIRP ES masks)

ntc_id=123456789, scen_id=1, seq_no=1, e_as_id=-1, orb_id=-1, sat_orb_id=-1, mask_id=1
ntc_id=123456789, scen_id=2, seq_no=1, e_as_id=-1, orb_id=-1, sat_orb_id=-1, mask_id=1

(Same mask is applicable to all configurations and user uplink scenarios)

ntc_id=123456789, scen_id=3, seq_no=1, e_as_id=-1, orb_id=-1, sat_orb_id=-1, mask_id=2

#### sat_lnk Table (Optional - if partial constellation used)

# If scenario 1 only uses first 30 satellites in each plane:
ntc_id=123456789, scen_id=1, orb_id=1, sat_orb_id=1, seq_no=1
ntc_id=123456789, scen_id=1, orb_id=1, sat_orb_id=2, seq_no=2
...
ntc_id=123456789, scen_id=1, orb_id=1, sat_orb_id=30, seq_no=30
[Repeated for orb_id=2 through 10]

## Appendix 1B: Migration Notes from Legacy Structure

### B.1 Mapping from Legacy Tables

| Legacy Table | Legacy Field | New Table | New Field | Notes |
| --- | --- | --- | --- | --- |
| non_geo | sat_name | epfd_param | scen_name | Now scenario-specific |
| non_geo | nbr_sat_td | epfd_param | nbr_sat_td | Direct mapping |
| non_geo | density | epfd_param | density | Direct mapping |
| non_geo | avg_dist | epfd_param | avg_dist | Direct mapping |
| non_geo | f_x_zone | epfd_param | f_x_zone | Direct mapping |
| non_geo | x_zone | epfd_param | x_zone | Direct mapping |
| grp | elev_min | epfd_param | elev_min | Now scenario-specific |
| grp | freq_min, freq_max | epfd_freq | freq_min, freq_max | Multiple ranges supported |
| orbit | apog+apog_exp | orbit | apog_km | Simplified format |
| orbit | perig+perig_exp | orbit | perig_km | Simplified format |
| orbit | op_ht+op_ht_exp | orbit | op_ht_km | Simplified format |

### B.2 Key Changes

- Scenario-based organization: Parameters previously global are now scenario-specific

- orbit_set_id added: Enables multiple orbital configurations

- Simplified altitude parameters: No exponent fields needed

- Enhanced linking: More flexible mask and satellite associations

## Appendix 1C: Reference to S.1503-4 Sections

### C.1 Database Parameter Mapping

| S.1503-4 Section | Parameter | Database Location |
| --- | --- | --- |
| §B3.1 | N_sat | orbit.nbr_sat_pl (sum across planes) |
| §B3.1 | H_MIN | orbit.op_ht_km |
| §B3.1 | DoesRepeat | orbit.f_stn_keep |
| §B3.1 | W_delta | orbit.keep_rnge |
| §B3.1 | MIN_EXCLUDE | epfd_param.x_zone or XML |
| §B3.1 | MIN_DURATION | XML (operating parameters) |
| §B3.1 | MAX_CO_FREQ | XML (operating parameters) or sat_oper.nbr_op_sat |
| §B3.1 | ES_DENSITY | epfd_param.density |
| §B3.1 | ES_DISTANCE | epfd_param.avg_dist |
| §B3.2 | A[N] | orbit: calculated from apog_km, perig_km |
| §B3.2 | E[N] | orbit: calculated from apog_km, perig_km |
| §B3.2 | I[N] | orbit.inclin_ang |
| §B3.2 | O[N] | orbit.long_asc |
| §B3.2 | W[N] | orbit.perig_arg |
| §B3.2 | V[N] | phase.phase_ang |

### C.2 Calculation References

- Semi-major axis (a): a = R_e + (apog_km + perig_km)/2

- Eccentricity (e): e = (apog_km - perig_km)/(apog_km + perig_km)

- Where R_e = 6378.145 km (Earth radius)