| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 4A/TEMP/262 | Annex 21 to Document 4A/1064-E |
|  | 28 May 2026 |
|  | English only |
| Annex 21 to Working Party 4A Chair’s Report |  |
| STAGE 1 PROCESS (RESOLUTION 76 (REV.WRC-23)) |  |
| Text elements for the Chair’s Report on the development of the Stage 1 Process (Consultation Meeting Trigger) as required under Resolution 76 (Rev.WRC-23) |  |

[Editor’s Note: The material below has not been fully reviewed nor agreed at this stage remains under discussion in WP 4A. As this is a work in progress, all revisions marks, comments, and highlights should be preserved in this document until its finalization.]

[Editor’s Note: The aim of this document is to concisely describe the Process for Stage 1, preferably in three to five pages.]

[Editor’s Note: At the end of the May 2026 meeting of WP 4A, there remained different views as to the best method to take for the calculations needed under Stage 1. As further edits to this section depend on setting a method, it will be a priority to address at the next meeting of WP 4A. The methods for Stage 1 that have been discussed are:

- Dispense with Stage 1;

- Use the methodology of Stage 2;

- Using Recommendation ITU-R S.1503, all steps of the existing algorithm for the most recent in-force version of S.1503 should be used. The results of the in-force examination of S.1503 should be convolved with other results of the S.1503 examination for the various non-GSO filings.  AS an example, if the algorithm of S.1503 only includes examination of the WCG, then the Stage 1 process should perform a convolution of the WRC results of each non-GSO system.

- First develop a recommendation that contains a methodology based on Recommendation ITU-R S.1503 epfd down algorithm (section D5.1) referred to as the “Step-1 methodology”. This methodology should be based on a grid of points defining a set of representing geometry. The grid of points should use the longitude independency of S.1503 to reduce the number of test points. This work should be concluded as soon as possible and, while the output of this methodology should not be used to adapt the operations of non-GSO systems to comply with the aggregate power levels it could be considered as the technical justification to trigger, if required, the consultation meeting referred to in resolves 3 of Resolution 76 (Rev.WRC-23).

Consideration of these and other options will continue at the next meeting of WP 4A.]

Below is a simplified Process for estimating the aggregate equivalent power flux density levels into geostationary satellite networks produced by multiple non-GSO FSS systems operating co-frequency in the frequency bands subject to Resolution 76 (Rev.WRC-23). The results generated by this simplified process are to trigger Stage 2.

This document contains the simplified Process that may be used to estimate the aggregate equivalent power-flux density (epfd) produced by multiple non-GSO FSS systems operating co-frequency in each of the frequency bands subject to Resolution 76 (Rev.WRC-23). Note that this process does not accurately model non-GSO operations; therefore, it does not constitute the methodologies called for by Resolution 76 (Rev.WRC-23), resolves 3 (Rev.WRC-23).

Step 1	[Version 1] The objective of this first step is to identify the operational space stations of every relevant non-GSO system that should be taken into account in the assessment and any associated technical characteristics needed to compute statistics of the epfd generated at a certain GSO geometry. Such information should be provided by the notifying administrations of relevant non-GSO systems.

That could be, for example, to send a reminder to that administration to provide such data. This could be part of the Terms of Reference to be developed.

[Version 2] The objective of this first step is to identify the operational space stations of every relevant non-GSO system that should be taken into account in the assessment and any associated technical characteristics needed to compute statistics of the epfd generated at a certain GSO geometry. Such information needs to  be provided  by the notifying administrations of relevant non-GSO systems or, failing that, [by a credible third party based on publicly available data].

Operational space station information and technical characteristics of non-GSO systems shall be submitted by the notifying administration as a matter of priority; if not submitted by the notifying administration, third parties may supplement based on publicly available data, provided that: 1) Third-party data shall indicate the source and data collection time; 2) Core data involving orbital parameters, transmit power, etc., shall take effect only after written confirmation by the notifying administration; 3) Third-party data without confirmation shall not be used as the basis for determining EPFD exceedance.

The calculated output of aggregate epfd from the previous consultation meeting should be used as a reference situation for the next sub-sequent meeting. Figure 1 contains a flow-chart showing the overall process for assessing a potential exceedance of epfd limits contained in Tables 1A to 1D of Resolution 76 (Rev.WRC-23) and how the output from one consultation meeting should be used as an input to the sub-sequent consultation meeting.

For each of the applicable frequency band(s) identified in Tables 1A to 1D of Resolution 76 (Rev.WRC-23), the information should be organised in two databases: the “Mask database” and the “SRS database”, respectively:

–	the Mask Database should include any data specified in Section B4 of the most recent version of Recommendation ITU-R S.1503; and

–	the SRS Database should include any remaining data necessary to apply the Process included in Recommendation ITU-R S.1503, including the association between those pfd/e.i.r.p. masks included in the abovementioned Mask Database and each of the identified operational space stations.

The Mask and SRS databases are intended for use by the proposed Stage 1 process only. These datasets may differ from the databases submitted by administrations for Radio Regulations (RR) Article 22 epfd examinations.

FIGURE 1

Flow-chart of the aggregate epfd compliance evaluation process

Step 2	The algorithm contained in Recommendation ITU-R S.1503 should be used to generate epfd statistics for each of the analysed non-GSO systems. An assessment should be carried out for a range of GSO geometries. Table 1 is summarizing the number of unique geometries for various grid sizes.

TABLE 1

Summary of GSO geometries

| Latitude range | Latitude granularity | Longitude range | Longitude granularity | GSO delta-lon range | GSO delta-lon granularity | Summed geometries for one GSO earth station antenna size |
| --- | --- | --- | --- | --- | --- | --- |
| 75° S-75° N | 1° | 180° W-179° E | 1° | 0°-75° | 1° | 3 658 968 |
| 75° S-75° N | 2.5° | 180° W-177.5° E | 2.5° | 0°-75° | 2.5° | 238 953 |
| 75° S-75° N | 5° | 180° W-175° E | 5° | 0°-75° | 5° | 30 960 |
| 70° S-70° N | 10° | 180° W-170° E | 10° | 0°-70° | 10° | 3 960 |
| 75° S-75° N | 15° | 180° W-165° E | 15° | 0°-75° | 15° | 1 248 |

A grid granularity of 15° should provide a good set of various GSO geometries  while avoiding computation complexity. If the non-GSO system operates with symmetrical performance with no variability in longitude nor across the northern and southern hemispheres in latitude, then considering 15° step size in latitude and GSO delta-longitude, the 1248 geometries from Table 1 result in 29 unique GSO geometries which could be repeated for the rest of the grid.

Each non-GSO system should therefore be evaluated over a grid organised as follows:

–	Latitude every 15° between −75° N and 75° N

–	Longitude every 15° between −180° E and 165° E

–	Each grid point would have GSO delta-longitude from 0° to 75° spaced by 15°.

The epfd statistics for each GSO geometry should be gathered in tables of probability density functions (PDF) and cumulative distribution functions (CDF) with bin size resolution equal to 0.1 dB. The results should be summarized into XML format with the following structure:

[Editor’s Note:  units of PDF value should be clarified.]

<satellite_system ntc_id=”NTC-ID” sat_name=”SAT-NAME”>

<pdf_results a_name=”lat_es” b_name=”lon_es” c_name=”lon_gso” d_name=”pdf” type=”pdf_table” low_freq_mhz="freq_min" high_freq_mhz="freq_max" refbw_khz="40 or 1000"

<by_a a=”75”>

<by_b b=”-180”>

<by_c c=”0”>

<pdf d=”EPFD_BIN1”>PDF_VALUE</pdf>

<pdf d=”EPFD_BIN2”>PDF_VALUE</pdf>

</by_c>

</by_b>

</by_a>

</satellite_system>

The CDF results is provided in the same format, but with exchanging “pdf” to “cdf”.

If Step 2 is applied for preparation to not the first consultation meeting, then only new non-GSO satellites need to be assessed over the grid.

Step 3 	For each of the GSO geometries considered, the PDF outputs of Step 2 for all relevant non-GSO systems should be convolved together to provide one set of aggregate epfd PDF which should be converted to one CDF per GSO geometry and per antenna size. The output of Step 3 will consist in as many aggregate epfd CDF curves as the number of GSO geometries specified in Step 2 above multiplied by the number of antenna sizes specified in Tables 1A to 1D of Resolution 76 (Rev.WRC-23).

Except for preparation to the first consultation meeting, the PDFs resulting from new operational non-GSO satellites should be convolved with the PDF reference situation based on existing non-GSO satellite systems and then converted to new updated CDF reference situation.

[Editor’s Note:  Need to consider how to address changes to an existing system that impact the reference situation.]

Step 4 	Each of the aggregate epfd CDF curves determined through Step 3 should be compared with the appropriate aggregate epfd limits specified in Tables 1A to 1D of Resolution 76 (Rev.WRC-23). If, for any percentage of time, the epfd CDF curve exceeds the relative value of the aggregate epfd limits, it can be concluded that, for the particular GSO geometry considered, the aggregate epfd limits specified in Resolution 76 (Rev.WRC-23) may be exceeded needing further attention through Stage-2 Process.

[Editor’s Note: The material below is temporarily maintained in anticipation that portions of which may be needed for insertion in the Process above so as to facilitate its correct understanding.]

1	that the Process specified in Annex 1 may be used to estimate the aggregate co-frequency epfd produced by multiple non-GSO FSS systems operating in frequency bands subject to Resolution 76 (WRC-23);

2	that the aggregate epfd calculations should involve two assessments, one considering only the operational space stations of non-GSO FSS systems and another, for information only, considering also non-GSO FSS space stations planned to be deployed within twelve months (see resolves 8 of Resolution 76 (WRC-23));

3	that, in the event that the aggregate interference levels determined with the Process specified in Annex 1 shows a potential exceedance of the interference levels given in Tables 1A to 1D of Resolution 76 (WRC-23), administrations operating or planning to operate non-GSO FSS systems and/or GSO networks should hold, as a matter of urgency, a consultation meeting with the objective to determine, through the use of a Process able to accurately model the operations of such non-GSO FSS systems, whether the interference levels given in Tables 1A to 1D of Resolution 76 (WRC-23) are exceeded and, in such a case, to adapt the operation of co-frequency non-GSO FSS systems in the frequency bands referred to in Tables 1A to 1D of Resolution 76 (Rev.WRC-23) to ensure that such levels are complied with;

4	that, in the event that any of the methodologies needed by the consultation meetings to implement resolves 3 above is not available, the first consultation meeting(s) should focus on finalizing, as a matter of urgency, such methodologies;

5	that, in order to assist administrations in implementing resolves 4 above, the Terms of Reference indicated in Annex 2 could be considered;

6	that recognizing that the conservative nature of the Recommendation ITU-R S.1503 and its input parameters does not represent the actual operations of non-GSO systems, the results obtained with the Process specified in Annex 1 are for information only and should not be used as evidence of exceedance of the aggregate epfd limits nor to adapt the operation of co-frequency non-GSO FSS systems in the frequency bands referred to in Tables 1A to 1D of Resolution 76 (Rev.WRC-23).

_______________

____________________

For the purpose of this Recommendation, a non-GSO system is considered “relevant” if either at least one of its space stations is deployed or its notifying administration certifies that at least one of its space stations will be deployed within [twelve] months (see recommends 2).

For the purpose of this Recommendation, a non-GSO system is considered “relevant” if either at least one of its space stations is deployed or its notifying administration certifies that at least one its space stations will be deployed within [twelve] months (see recommends 2).

Publicly available sources include: NORAD database and notifications published on the ITU website and/or in the BR IFIC.